Prep the Pi (64-bit OS recommended)

sudo apt update

sudo apt install -y python3-venv python3-pip ffmpeg libsndfile1

python3 -m venv beeenv

source beeenv/bin/activate

python -m pip install --upgrade pip

2) Install Python deps (ARM/aarch64-friendly)

Create requirements_pi.txt:

numpy==1.26.4

scipy==1.11.4

scikit-learn==1.3.2

joblib==1.3.2

librosa==0.10.1

soundfile==0.12.1

audioread==3.0.1

Then:

pip install -r requirements_pi.txt

Tip: Use librosa ≥0.10 (no numba dependency headaches on Pi).

3) Copy your artifacts to the Pi

Put these in, say, /home/pi/queenbee/:

- queen_bee_model.pkl (from your laptop training)

- enhanced_queen_bee_detection.py (the windowized inference script you made earlier)

If you don’t have the script yet, use the one we discussed (loads the .pkl, does 1s window MFCC+Δ+Δ² features, scales → selects → predicts, then aggregates across windows with mean_proba or max_proba).

IMPORTANT: In enhanced_queen_bee_detection.py, set the constants to match how you trained:

SR = 22050

WINDOW_SECONDS = 1.0   # or 1.0 if you trained on 1s windows

HOP_SECONDS    = 0.5   # or 0.5 for 50% overlap, etc.

4) Run a prediction on the Pi

source beeenv/bin/activate

cd ~/queenbee

python enhanced_queen_bee_detection.py /path/to/recording.wav --model queen_bee_model.pkl --agg max_proba

Example output:

Prediction: queen_present | Confidence: 0.87

5) (Optional) Record audio on the Pi, then run

With a USB mic:

arecord -l                     # list devices

arecord -D plughw:1,0 -f S16_LE -r 22050 -c 1 -d 10 test.wav

python enhanced_queen_bee_detection.py test.wav --model queen_bee_model.pkl --agg max_proba